# Fabrication and Integration Process

### 1. Material Choice Justification :

- MDF was selected for the primary structure due to its rigidity, stability, and ease of fabrication, making it well-suited for housing internal wiring and supporting mechanical components like the linear actuator. Its weight also helps anchor the system, reducing unwanted vibrations during operation.

- Acrylic was used for the upper enclosure walls to provide a transparent finish for aesthetics that enhances visibility of the internal mechanisms while maintaining a lightweight structure.

- Mirrors were incorporated for their high reflectivity and surface precision, enabling controlled redirection of the laser beam across multiple points in the maze.

- PLA (3D Printing) was used to fabricate custom connector components, specifically to couple the DC motor shaft with a wooden dowel in the swing rotation mechanism. PLA was chosen for its ease of prototyping, strength for low-load connections, and ability to produce precise, customized geometries quickly.

2. Fabrication Steps :

- Design & Laser Cutting (MDF Structure) — Precise 2D CAD files were developed for the ride base box and internal linear actuator housing. Components were laser cut from MDF, followed by 45° edge sanding to ensure clean joints and accurate assembly. Surface finishing was then done.

- Acrylic & Mirror Fabrication — Separate 2D files were created for cutting acrylic panels and mirrors to exact dimensions. Acrylic surfaces were intentionally heat-treated using a heat gun to induce controlled bubbling, creating a textured visual effect.

- 3D Modelling & Printing (PLA Connectors) — Custom connector components were designed with precise measurements to couple the DC motor and stepper motor shafts to a wooden dowel used in the spinning mechanism. These parts were 3D printed in PLA for accuracy and fit.

- Assembly & Integration — All structural, mechanical, and electronic components were assembled, including alignment of mirrors for the laser path and integration of internal mechanisms.

- Final Finishing & Detailing — Decorative and aesthetic enhancements were applied to complete the model.

- Top of Form

- Bottom of Form